| Catheter | Strain relief | Luerlock connector |
| --- | --- | --- |

- Strain relief prevents kinking at the connection between the catheter and luerlock connector

- Closed-tip catheter

- Distance to first marker = 11cm (needle length)

- 5cm between markers

Lumbar CSF Drain – Medtronic Catheter

| If guidewire will be used, pre-flush catheter with saline before inserting guidewire - Connect luerlock connector to catheter - Flush with saline |  |
| --- | --- |
| Threading guidewire into the catheter: - Loosen the luer cap on the adjustable stop - Thread wire through adjustable stop out the rounded end and into the catheter (via the luer connector) to the catheter tip - Tighten the luer on the adjustable stop to affix the stop to the guidewire |  |
| After catheter is insitu |  |
| - Slide strain relief over catheter |  |
| - Connect luerlock connector w/ integral plug to catheter |  |
| - Slide strain relief up to snugly fit with the luerlock connector w/ integral plug |  |
| - Suture in the groove of the strain relief to secure it to the catheter (0-silk recommended) |  |